F5 Configuration Standardization & Validation Plan

(Designed for 300+ BIG-IPs, Jira-driven, automation-ready)

Sanket, for 300 F5s the only scalable way is:

- Phase 1: Define clear configuration domains + parameters and map each to Jira tickets.

- Phase 2: Automate config extraction and comparison against a “golden standard” per domain.

Below is:

- A parameter-wise breakdown of F5 config into logical sections that can map 1:1 or 1:few with Jira tickets.

- A deep-dive Phase 2 plan: how to fetch configs, validate against standards, and eventually automate.

Phase 1 – Configuration Domains and Jira Sections

Think in terms of “domains” that can be independently validated and owned by different tickets / assignees.

1. Platform & System Baseline

Jira Section: “BIG-IP System & Platform Baseline”

Scope per device (or HA pair):

- Software & licensing

  - BIG-IP version, build, HF level (target standard version per environment).

  - Licensed modules (LTM, ASM, APM, AFM, DNS, etc.).

  - Provisioning level of each module (none / minimum / nominal / dedicated).[1][2]

- System identity

  - Hostname and device name.

  - Platform type (VE / hardware; series).

- System global settings[3]

  - sys global-settings:

    - hostname

    - mgmt-dhcp, lcd-display, gui-setup

    - mgmt-https / mgmt-https-allow-ipv6

  - sys db keys you care about (e.g. disabling bash, GUI timeout, etc.).

Each of these becomes line items in your “standard config” doc and a checklist in a Jira ticket.

2. Management Access & Hardening

Jira Section: “Management Plane Security & Access Control”

Parameters:

- Management access

  - mgmt IP/subnet and GW.

  - Allowed management protocols: HTTPS, SSH, disabled Telnet.[4][5]

  - GUI and SSH idle timeouts.

  - Allowed IP ranges (mgmt ACLs / firewall).

- Users & auth

  - Local admin accounts & password policy.

  - Remote auth (TACACS+/RADIUS/LDAP):

    - AAA servers, timeouts, fallback behavior.

    - Role mapping (Admin / Operator / Guest).

- Management SSL / SSH security

  - HTTPS cipher suites and TLS versions allowed on management.

  - SSH ciphers, MACs, KEX algorithms.

  - Login banner / legal notice.

One Jira ticket can be “Verify management plane configuration on all BIG-IPs matches standard.”

3. Network Layer (Data Plane Underlay)

Jira Section: “Network: VLANs, Self IPs & Routing”

Parameters to standardize and validate per device/cluster:

- VLANs[6][4]

- VLAN names and tagging conventions (e.g. `ext_<zone>`, `int_<zone>`, `ha_<cluster>`).

  - Tagged/untagged interfaces per VLAN.

  - MTU (standard vs exceptions).

- Self IPs[5][7]

  - One self IP per VLAN, clear naming.

  - traffic-group (for HA).

  - Port lockdown mode (Allow Default / Allow None / Allow Custom).[4]

    - Your standard should be “Allow None” + explicit services for production.

- Routes

  - Default route (standard per environment).

  - Static routes for back-end networks.

  - Dynamic routing protocol usage (if any) and standard params.

- SNAT address objects (if using SNAT pools).

Separate tickets could be:

- “Validate VLAN & self IP standards”

- “Validate routing standards”

4. Device Service Clustering / HA

Jira Section: “High Availability & Device Service Clusters”

Parameters:[8][6]

- Device group configuration

  - Device group type: sync-failover vs sync-only.

  - Membership: correct devices in each group.

  - Auto-Sync enabled/disabled (your decision).

  - Full sync success / “In Sync” state.

- Failover settings

  - Network failover enabled.

  - Failover unicast addresses (mgmt / self IPs).

  - Redundant config for HA VLAN (both units, consistent addressing).[6]

- Traffic groups

  - traffic-group-1 assigned to correct floating objects:

    - Floating self IPs.

    - Virtual addresses.

  - If multiple traffic groups: clear ownership strategy.

- Mirroring

  - Primary / secondary mirror IPs defined.

  - Mirroring VLAN.

- HA Groups (if used)[8]

  - Group definitions

  - Weight assignments

  - HA score thresholds.

A single Jira ticket per device group is realistic here.

5. System Services: DNS, NTP, Logging, SNMP

Jira Section: “System Services: DNS, NTP, SNMP & Logging”

Parameters:[9][10]

- DNS (sys dns)

  - DNS servers (on corporate standard list).

  - Search domains.

- NTP (sys ntp)

  - NTP servers and timezone.

  - NTP sync status baseline.

- SNMP

  - Read-only communities (or v3 users).

  - Trap destinations and standard MIBs.

  - Contact and location fields.

- Syslog / logging

  - Remote syslog targets (SIEM).

  - Local log levels (LTM, ASM/APM/AFM if in use).

  - Log rotation size and retention.

This could be 1 Jira section, with sub-tasks per service (DNS, NTP, SNMP, Syslog).

6. LTM Profiles – Global Traffic Behaviour

Profiles are the heart of LTM standardization.[11][2]

Jira Section: “LTM Standard Profiles (TCP/HTTP/SSL/etc.)”

Profiles to define and then validate:

- TCP profiles[11]

  - Separate client-side and server-side profiles.

  - Standard values:

    - Nagle, slow-start, delayed ACK, window scaling.

    - Idle timeout.

    - DSACK / selective ACK.

    - congestion control (newreno, cubic, etc.).

- HTTP profile(s)[12][11]

  - One or more standard HTTP profiles:

    - X-Forwarded-For header insertion.

    - OneConnect usage.

    - Enforced HTTP versions.

    - Compression on/off.

    - Response headers handling (server header masking, etc.).

    - Redirect/fallback host behavior.

- SSL profiles (ClientSSL / ServerSSL)[13][12][11]

  - Cipher suites and order.

  - Min/Max TLS versions (e.g. 1.2–1.3).

  - Renegotiation policy.

  - Session resumption.

  - OCSP/CRL settings.

  - Key sizes and cert types.

- Persistence profiles[14][15][3]

  - Standard cookie persistence profile (cookie name, hash method).

  - Source address persistence (timeout, IPv6 mask).

  - SSL session ID persistence (where needed).

- Other profiles as required

  - FastL4 profile (for L4-only VIPs).

  - HTTP compression profile.

  - Web acceleration profile.

For Jira:
1 ticket = “Create and document standard LTM profiles.”
Later tickets per device/app = “Ensure only approved standard profiles are attached unless exception is documented.”

7. Health Monitors

Jira Section: “Health Monitoring Standards”

Parameters per monitor & per pool:[2][13]

- Monitor objects

  - Type: tcp, http, https, icmp, gateway_icmp, radius, external, etc.

  - Interval and timeout (ratio; e.g. 5/16).

  - Send/Receive strings for HTTP/HTTPS/custom.

  - Manual resume or automatic.

  - Up/down thresholds (for advanced monitors).

- Monitor assignment

  - Every pool has at least one monitor.

  - No use of none monitor for production.

  - For application types: e.g. HTTP GET /health or /status instead of just TCP.

This maps cleanly to 1 Jira ticket per environment: “Standardize and validate monitors.”

8. Virtual Servers (Per-FQDN Application Entry Point)

This is where per-application FQDN standards live.

Jira Section: “Virtual Server Standards (per Service / FQDN)”

Parameters per VIP:[7][2]

- General

  - Naming convention: includes env, app, port.

  - Type: Standard / Performance (Layer 4) / Forwarding (IP).

  - Destination IP and port.

  - address-translation / port-translation behavior.

  - Enabled/Disabled state policy.

- SNAT & Source IP

  - SNAT type: None / Automap / SNAT pool.

  - Standard SNAT behavior for:

    - Internet-facing vs internal-only.

    - RADIUS, SIP, etc. (where SNAT may break protocols).[16]

- Profiles attached

  - Client TCP, server TCP.

  - HTTP, OneConnect.

  - ClientSSL / ServerSSL (if HTTPS/SSL).

  - Persistence profile.

  - FastL4 profile (if L4).

- Traffic restrictions

  - VLANs / tunnels allowed.

  - Source address filters (allow-list / deny-list).

  - Connection limit, connection rate limit.

- Policies & iRules

  - Which LTM policies are allowed (e.g. host-based routing, header rewrite).

  - Approved iRules list (no adhoc copies).

You can have:

- 1 Jira ticket template per virtual server (generated automatically later),

- Or batch tickets like “Audit all internet-facing HTTPS VIPs in environment X.”

9. Pools & Pool Members

Jira Section: “Pool & Member Standards”

Parameters per pool:[13][2]

- Pool configuration

  - Naming convention.

  - Load balancing method (Round Robin, Least Connections, Ratio, etc.) – standard per type.[15][14]

  - Priority group activation policy (if using priority groups).[17][15]

  - Min active members / reselect policy.

  - Slow ramp time (for newly added nodes).

  - Max connections (per pool or member).

- Members

  - Only IP:port (no DNS in pool unless required).

  - Consistent ports for same app.

  - Monitor assignment at pool-level (not per-member, unless truly needed).

  - Admin up/down state for maintenance.

10. iRules & Local Traffic Policies

Jira Section: “Custom Logic – iRules & Local Traffic Policies”

Parameters:[18][19]

- iRules

  - Naming standards (ir_appname_function_env).

  - Versioning and repository alignment (Git).

  - Restricted set of “approved” generic iRules (logging, security headers, redirects).

  - Where attached: which VIPs may use which iRules.

- Local Traffic Policies

  - Central policies for:

    - URL-based routing.

    - Host header routing.

    - Maintenance mode redirects.

Each iRule / policy should be linked to a change control and stored in source control.

11. Application Security (ASM/WAF, AFM, DDoS) – If Licensed

Jira Section: “Application & Network Security Policies”

Parameters:[20][21]

- WAF/ASM

  - Security policy mode (transparent / blocking).

  - Policy baseline per app type:

    - Signature sets.

    - Bot detection / schema validation (for APIs).[21]

  - Logging profile and log destination.

- AFM / DDoS

  - Network firewall policies per VLAN / route domain.

  - DDoS thresholds and protections.

This is typically owned by the security team; separate Jira epics per policy family.

12. Telemetry, Backup & Automation

Jira Section: “Backups, Telemetry & Automation Hooks”

Parameters:[22][23][24][9]

- Backups

  - UCS schedule (daily/weekly).

  - SCF export policy.

  - Off-box backup location.

- Telemetry

  - Telemetry Streaming or custom syslog/HSL to SIEM.

  - Metrics to central monitoring (CPU, TMM, pool status).

- Automation state

  - Whether device is:

    - Purely manual.

    - Ansible-managed.

    - AS3/DO/TS-managed.

Later, full automation can become source-of-truth.

Phase 2 – Deep-Dive: How To Fetch, Validate & Standardize at Scale

Now, how to actually do this for 300 boxes and multiple FQDNs.

1. Define your “Standard” as Data, Not Just a Document

For each domain above, define a machine-readable standard: YAML/JSON.

Example (pseudo-YAML):

global:
          ntp_servers:
            - 10.10.10.10
            - 10.10.10.11
          dns_servers:
            - 192.168.1.10
            - 192.168.1.11
        
      profiles:
          tcp:
            client_standard:
              idle_timeout: 300
              nagle: disabled
              dsack: enabled
          http:
            standard:
              insert_x_forwarded_for: true
              oneconnect: true
        
      ssl:
          clientssl:
            min_tls_version: "1.2"
            max_tls_version: "1.3"
            cipher_string: "DEFAULT:!EXPORT:!MD5:!RC4"

This same structure later drives validation scripts and also your Jira “standard config” document.

2. Choose Access Mechanism: iControl REST vs tmsh vs Ansible

For 300 devices, avoid manual login.

Recommended:

- Primary: iControl REST (HTTPS) – scriptable via Python; stable.

- Optional: Ansible modules (bigip_device_info, bigip_*) to pull facts and compare.[23][24][18]

You can still use tmsh commands under the hood (list ltm virtual, etc.) but via automation.

3. What To Fetch for Each Domain

Map domains → concrete data sources.

Examples (tmsh style; REST has equivalent endpoints):[25][22]

- System & Platform

  - tmsh list sys global-settings

  - tmsh list sys ntp

  - tmsh list sys dns

  - tmsh list sys snmp

  - tmsh show sys version

- Network

  - tmsh list net vlan

  - tmsh list net self

  - tmsh list net route

- HA & Device Groups

  - tmsh list cm device

  - tmsh list cm device-group

  - tmsh list cm traffic-group

- Profiles & LTM

  - tmsh list ltm profile tcp

  - tmsh list ltm profile http

  - tmsh list ltm profile client-ssl

  - tmsh list ltm profile server-ssl

  - tmsh list ltm persistence

  - tmsh list ltm monitor

  - tmsh list ltm virtual

  - tmsh list ltm pool

- Security

  - tmsh list security firewall

  - tmsh list asm policy (if ASM)

Before automating, tmsh load sys config verify can validate syntax but not “standard vs nonstandard”.[25]

4. Validation Approach: High-Level Design

A good pattern for you as a Network Security Automation Engineer:

- Inventory

  - Maintain a CSV/JSON of all devices:

    - mgmt IP, HA pair group, environment, owner, etc.

- Collector Script (Python)

  - Loops through devices.

  - Queries iControl REST endpoints (or runs tmsh via SSH).

  - Normalizes data into a fixed schema per domain.

  - Stores results in JSON files or a central DB.

- Validator

  - Reads:

    - Collected config.

    - Golden-standard JSON/YAML.

  - For each parameter:

    - Compare actual vs expected.

    - Classify as:

      - Match

      - Allowed exception (from a separate exceptions list)

      - Deviation.

- Output

  - Generates:

    - CSV report with rows like:

      - device, domain, object_name, parameter, expected, actual, status

    - Optional:

      - Preformatted text for Jira ticket description.

- Integration with Jira

  - For each device/domain with deviations:

    - Create/update Jira ticket via REST API.

    - Attach CSV / summary table.

This is pure “infrastructure as data”; AS3/DO can be a future step where F5 is fully declarative.[24][23]

5. Suggested Domain-by-Domain Validation Strategy

Because 300 devices is huge, go in waves.

Wave 1 – Low-Risk, Global Stuff

- System services: DNS, NTP, SNMP, Syslog.

- Logging & backups.

Reasons:

- Safe to change.

- Easy to standardize.

- Limited app impact.

Validator logic sample (DNS):

- From sys dns, read name-servers.

- From standard, read allowed_dns_sets (maybe per environment).

- If device’s list != one of allowed sets → flag.

Wave 2 – Network & HA

- Validate VLAN names, self IP format, port lockdown policy.

- Validate HA device groups and traffic groups.

Most of this affects control-plane / failover behavior; treat carefully but still central.

Wave 3 – Profiles & Monitors

- Compare all ltm profile http where parent != default:

  - Identify non-standard derived profiles.

- Compare ltm profile tcp against your standard baseline.

- Compare monitors (interval/timeout, type) by tag (app type).

Here, you begin to limit proliferation of random custom profiles created by different engineers.

Wave 4 – Application Objects (Virtuals & Pools)

- Per FQDN / per VIP:

  - Verify that the attached profiles are on the approved list.

  - Validate SNAT settings vs environment policy.

  - Validate load balancing method vs standard.

At this point, app owners must be brought into the discussion; not everything can be forced into a rigid template.

6. Handling Exceptions Cleanly

There will always be exceptions (e.g. RADIUS VIP that can’t use SNAT, legacy app needing weaker ciphers).

Create an exceptions file:

exceptions:
          - device: bigip-prod-01
            object_type: ltm virtual
            object_name: /Common/vs_legacy_app_443
            parameter: clientssl_profile
            reason: "Legacy Java 6 client; requires TLS1.0"
            approved_by: "SecurityTeam"

Validator behavior:

- If mismatch but listed in exceptions → status EXCEPTION_APPROVED instead of NON_COMPLIANT.

This is necessary to avoid Jira noise.

7. Mapping Back to Jira - Practical Layout

A workable Jira structure for you:

- Epic: “BIG-IP Configuration Standardization – Phase 1 & 2”

Under this Epic, create Stories per domain:

- F5-SYS-STD – Define & document system/global standards.

- F5-NET-STD – Define & document network/HA standards.

- F5-PROFILE-STD – Define baseline LTM profiles.

- F5-MON-STD – Define monitoring standards.

- F5-VIP-STD – Define virtual/pool standards.

Then for Phase 2, separate Stories:

- F5-COLLECTOR – Implement config collector (Python/Ansible).

- F5-VALIDATOR – Implement validator and reporting.

- F5-JIRA-INTEG – Integrate findings with Jira.

And Tasks under them:

- “Validate DNS/NTP on all prod BIG-IPs”

- “Validate HTTP/TCP profiles on all internet-facing VIPs”

- Etc.

Each task describes which parameters (from Phase 1 doc) must be checked.

8. Skill & Career Angle (Given Your Role)

This project is actually perfect for you:

- You’ll combine F5 expertise + Python/automation + governance.

- Good tools/tech around this:

  - requests + iControl REST.

  - Ansible with f5networks.f5_modules.

  - YAML standards tracked in Git.

- You can eventually move from “audit & fix” to “declarative push via AS3/DO”, which is the long-term automation model F5 is pushing.[23][24]

If you want to go deeper next, the following would be natural next steps:

- A concrete YAML standard template for one domain (e.g. HTTP/TCP/SSL profiles).

- A sample Python validator function for one profile type (say, ltm profile client-ssl), that you can plug into your environment and expand.

- Or a Jira ticket template text that you can reuse for each device/domain.

Happy to help you design the first validator for one object type (e.g. HTTP VIPs for your main web applications) and then you can replicate the pattern for the rest.

⁂

- https://techdocs.broadcom.com/us/en/vmware-cis/aria/aria-operations/8-18/vmware-aria-operations-load-balancing-guide-8-18-/f5-big-ip-ltm-installation-configuration.html

- https://techdocs.f5.com/content/kb/en-us/products/big-ip_ltm/manuals/product/ltm-basics-13-0-0/_jcr_content/pdfAttach/download/file.res/BIG-IP_Local_Traffic_Management__Basics.pdf

- https://techdocs.f5.com/kb/en-us/products/big-ip_ltm/manuals/product/bigip-system-essentials-11-6-0/1.html

- https://networkjutsu.com/f5-big-ip-ltm-ve-initial-configuration/

- https://techdocs.f5.com/kb/en-us/products/big-ip_ltm/manuals/product/bigip-system-ecmp-clustering-11-6-0/2.html

- https://rayka-co.com/lesson/f5-big-ip-ha-active-standby-configuration/

- https://clouddocs.f5.com/training/community/adc/html/class1/module1/lab1.html

- https://clouddocs.f5.com/training/community/adc/html/class6/Module1/Lab6.html

- https://my.f5.com/manage/s/article/K15783583

- https://my.f5.com/manage/s/article/K74134311

- https://my.f5.com/manage/s/article/K23843660

- https://www.uninets.com/blog/f5-ltm-local-traffic-profiles

- https://docs.oracle.com/en/middleware/standalone/coherence/15.1.1/develop-remote-clients/integrating-f5-big-ip-ltm.html

- https://www.f5.com/resources/white-papers/load-balancing-101-nuts-and-bolts

- https://wtit.com/blog/2019/05/27/understanding-f5-load-balancing-methods/

- https://community.cisco.com/t5/security-knowledge-base/how-to-cisco-amp-f5-deployment-guide-ise-load-balancing-using/ta-p/3631159

- https://www.youtube.com/watch?v=zf6eEQMoWa0

- https://clouddocs.f5.com/products/orchestration/ansible/devel/modules/bigip_config_module.html

- https://www.youtube.com/watch?v=uu-uTlTBB2g

- https://docs.cloud.f5.com/docs-v2/bot-defense/configure/waap

- https://www.youtube.com/watch?v=ELRmMDmCBbk

- https://my.f5.com/manage/s/article/K26582310

- https://www.f5.com/resources/white-papers/automating-f5-application-services-a-practical-guide-2019

- https://www.f5.com/resources/white-papers/automating-f5-application-services-a-practical-guide

- https://my.f5.com/manage/s/article/K53233552

- https://techdocs.broadcom.com/us/en/ca-enterprise-software/business-management/ca-service-management/17-3/administering/configure-ca-service-desk-manager/how-to-configure-the-f5-load-balancer-for-ca-service-desk-manager/prerequisites-for-configuring-the-f5-load-balancer.html

- https://clouddocs.f5.com/containers/latest/userguide/config-parameters.html

- https://cdn.studio.f5.com/files/k6fem79d/production/c7e0585d1c83a386e677dc6bd2899c49c6c75ebb.pdf

- https://docs.venafi.com/Docs/25.3/TopNav/Content/Drivers/t-F5LTMAdv-permissions-tpp.php

- https://www.f5.com/glossary/load-balancer

- https://www.certlibrary.com/blog/a-beginners-guide-to-configuring-f5-networks-big-ip-ltm/

- https://cloud.ibm.com/docs/framework-financial-services?topic=framework-financial-services-vpc-architecture-connectivity-f5-tutorial

- https://www.youtube.com/watch?v=MmFN4Pzigd8

- https://www.uninets.com/blog/f5-ltm-initial-setup-configuration

- https://techdocs.broadcom.com/us/en/vmware-cis/aria/aria-automation/8-13/on-prem-vra-8-load-balancing-guide-8-13/configuring-f5-big-ip-ltm.html

- https://docs.cyberark.com/mis-self-hosted/Docs/current/TopNav/Content/Drivers/r-F5LTM-AppObjSettings-tpp.php